3GPP TSG-SA WG3 LI Meeting #9 	Tdoc S3LI02_013

Amsterdam, Netherlands. 29 – 31 January 2002

23 January 2002

# Definition of the Type values for ULIC

## Spec:			33.108v0.3.0

## Release:		Rel-5

## Source: 		Nokia

## Document for:		Discussion. Source for a possible CR

## Summary

In the Aspen meeting #8, S3 LI group discussed three possible ways to encode the IEs in the ULIC (GLICv1) header:

- TLV

- ASN.1

- Both, TLV and ASN.1

This contribution offers the Type value definitions for the TLV coding.

Obviously, defining and maintaining TLV coded IE tables is times easier, than maintaining ASN.1 object tree. That’s the reason why TLV coded IEs are prevailing in 3GPP specifications.

For the past half a year, S3 LI has done a lot to define the ASN.1 description of IRI in 33.108. However, we still cannot finalize the specification, because we still do not have the ASN.1 object tree defined! The problem would not be the abstract syntax proper, but the need to co-ordinate the work with other standardization organizations. If S3 LI would have approved TLV as an alternative IRI parameter codin, we would have approved 33.108 by now.

## Proposal

Type values are already defined for the TLV coded header of a CC message, which is transferred by FTP.  It is proposed to use the same type values for ULIC header as well.

Acronyms in the ‘Presence of the IE in the CC message’ column ogf the table:

- M – stands for ‘Mandatory’

- R – stands for ‘ Recommended’

- C – stands for ‘Conditional’. Conditions shall be defined by a manufacturer

- O – stands for ‘Optional’

Table x. Type value definitions for the ULIC header

| IE name | Type of the IE | Length of the Value part | Value part of the IE | Presence of the IE in the CC message |
| --- | --- | --- | --- | --- |
| Header Length | 131 | 1 – 2 | The overall length of the ULIC header, or a CC message header. | R/C |
| Time Stamp | 134 | 4 – n | Actual value of a Time Stamp in the Value field of the IE in one of the following formats: Seconds since 1970-01-01 as in e.g. Unix (length: 4 octets); UTC, precision 1 second; UTC, precision 0.001 second. | R/C |
| Sequence Number | 141 | 1 – n | Actual value of a Sequence Number in the Value field of the IE. | M |
| Correlation Number | 144 | 8 – 20 | Actual value of a Correlation Number in the Value field of the IE. | M |
| MainElementID | 251 | 1 – 2 | Actual value of a MainElementID in the Value field of the IE | R/C |
| HeaderElementID | 252 | 1 – 2 | Actual value of a HeaderElementID in the Value field of the IE | R/C |
| PayloadElementID | 253 | 1 – 2 | Actual value of a PayloadElementID in the Value field of the IE | R/C |
| LIID | 254 | 1 – 25 | Actual value of a LIID in the Value field of the IE. | R/C |
| Private Extension | 255 | 1 – n | Actual value of a Private Extension in the Value field of the IE. | O |