3GPP TSG SA WG3LI	S3LI01-033

06-08 March, 2001

Tampa, USA

# Source:	Alcatel S.A.

# Title:	User Identities in IP multimedia domain

# Document for:	Discussion

The following contribution to SA#16 22-26 January 2001 in Los Angeles, USA addressing ‘User Identities’ was found.

This contribution was agreed and already incorporated in the latest draft of TS 23.228

It is suggested to make a split between public-user-identities and one private-user identity. The question is now can this be reused by SA3-LI for identifying targets in the IMS domain?

TSG-SA WG2	S2-010319
SA2#16 22-26 January 2001	User Identities
Los Angeles, CA

Source:		Drafting session

Title: 	Text for section 4.3.3 “User Identities”

Document for: 	Approval

# Proposal

If is proposed that the following text be included in TS23.228 Section 4.3.3.

4.3.3.1 Private User identities

Every IM subsystem subscriber shall have a user private identity. The private identity is assigned by the home network operator, and used, for example, for Registration, Authorization, Administration, and Accounting purposes.  This identity shall take the form of a Network Access Identifier (NAI) as defined in RFC2486.

Note: It is possible for a representation of the IMSI to be contained within the NAI for the private identity.

The Private User Identity is not used for routing of SIP messages.

4.3.3.2	Public user identities

Every IM subsystem subscriber shall have one or more public user identities [Ref 22.228].  The public user identity/identities are used by any user for requesting communications to other users.

Note:  For example, this might be included on a business card.

Both telecom numbering and internet naming schemes can be used to address users depending on the Public User identities that the user’s have.

The public user identity/identities shall take the form of SIP URL (as defined in RFC2543 and RFC2396) or E.164 numbers.

4.3.3.3 Routing of SIP signaling within the IP MultiMedia SubSystem

Routing of SIP signaling within the IMS shall use SIP URLs. E.164 format public user identities shall not be used for routing within the IMS, and session requests based upon E.164 format public user identities will require conversion into SIP URL format for internal IMS usage.

4.3.3.4	Relationship of Private and public user identities

The home network operator is responsible for the assignment of the private user identifier, and public user identifiers; other identities that are not defined by the operator may also exist.

Figure x. Relationship of the private user identity and public user identities

Editor’s Note:  Public User Identifier Portability issues are FFS.